Review8. The Processor

2019年6月2日

10:07

Hint: Bold - formula, underline - important, italic - not so important, grey - not so sure.

Implementation overview

  - Instruction count: ISA (Instruction set architecture) and compiler.

  - CPI and cycle time: CPU hardware.

- Basic MIPS architecture: math, memory access, branch and jump.

  - Implementation: memory, registers, ALU, and control logics, CPU operations.

- Overall implementation design:

- Two add units: first add unit increases PC value by 4; second add unit compute J-type instructions' address (offset * 4 + PC).

- Into data memory: memory instructions address after shamt, or arithmetic result.

- Into ALU: registers $rs, $rt from R-type instruction or immediate value from I-type instruction; data from arithmetic computation.

- Into register unit: new data or address from ALU.

Logic design basics

- Combinational elements: operate on data and output is a function of inputs.

- Sequential elements (state elements): store information.

  - Sequential without write control: PC, determined by posedge of clk.

  - Sequential with write control: data memory / register, updates on posedge of clk when write control is 1.

    - Data memory: <input> 32 bit address, 32 bit write data, 1 bit MemWrite, 1 bit MemRead; <output> 32 bit read data. - Read and Write.

    - Instruction memory: <input> 32 bit instruction address, 1 bit clk; <output> instruction. - Read only.

    - Registers: <input> 3 registers of 5 bit, 32 bit write data, 1 bit RegWrite; <output> 2 read data of 32 bit. - Read and Write.

- Units need clock: PC, instruction memory, registers, and data memory.

- Saved data in posedge of clk: PC (address of instruction), instruction, register, data, data memory address.

Detailed implementation for every instruction

- R-type instructions: read data from two registers, if RegWrite valid, then write data back to the third register.

- Input two register number of 5 bit and output 2 read data from registers of 32 bit, then input into ALU. Get result and back into write data. If RegWrite valid, write data into write register.

- Loads / stores instructions: lw and sw, two read registers (two read data), one write register and one write data.

- Right bottom is data memory, two controls are MemWrite and MemRead. ALU result goes into address, and read data 2 goes into write data.

- Input of ALU comes from two read data.

- J-type instructions: offset should be left shift 2 bits, so can be store 4 times more offset address. Therefore, before use, offset should be left shift 2 bits.

- Overall control signals:

- RegDst: assign the write register, $rd (1) or $rt (0).

- Branch: whether an instruction is a branch instruction.

- MemRead: control data memory to read data from address (lw).

- MemtoReg: choose source to write into register, memory-read-data (1) or ALU result (0).

- ALUOp: from function code to determine operation ALU should operate.

- MemWrite: control data memory to write data to address (sw).

- ALUSrc: determine the input of ALU, sign extend immediate value (1) or $rt (0).

- RegWrite: enable to write to registers.

Review9. The Overview of Pipeline

2019年6月2日

10:07

Pipeline design and performance

- Stages of pipeline: IF (instruction fetch), ID (instruction decode & register read), EX (execute ALU), MEM (memory access), WB (write back).

  - Left Reg means write, and right Reg means read.

  - Latency: time for each instruction.

Hazards

- Structure hazards: required resource is busy.

  - Hardware problem, use separate memories of instruction and data.

- Data hazards: wait for data read / write.

  - Forwarding: use result from ALU immediately into register read.

  - Load-use data hazard: use data from lw, solved by reorder.

- Control hazards: decisions depends on previous instruction.

  - Branch prediction: static (typical branch behavior) and dynamic (hardware record recent branch).

Review10. Instruction-Level Parallelism

2019年6月2日

10:07

Multiple issue

- Instruction level parallelism (LIP): deeper pipeline, or multiple issue.

- Static multiple issue by compiler and dynamic multiple issue by processor.

  - Static multiple issue: instructions into issue packets (very long instruction word, VLIW).

    - Hazards: data hazard split into two packets.

- Dynamic multiple issue (superscalar processor): CPU makes decision and no need for compiler scheduling.

- Speculation: guess and start operation as soon as possible.

  - Compiler reorder and hardware looks ahead for instructions (buffer results and flush on incorrect speculation).

  - Exceptions: static speculation add ISA support for deferring exceptions, dynamic speculation buffers exceptions until instruction completion.

Multiple issue scheduling

- Static multiple issue scheduling: reorder, dependencies between packets, pad with nop.

- An ALU / branch, load / store two packages multiple issue example:

- Replicate loop body: expand loop several times and use different registers (register renaming).

  - Anti-dependencies (name dependence): no data flow between two instructions, only because of the name of register.

- Hardware support, CPU executes out of order (commit result to registers in order).

- One IF / ID unit, several functional units, and one commit unit.

Review11. Large and Fast: Exploiting Memory Hierarchy

2019年6月2日

10:07

Memory

- DRAM (high bit density but poor latency) store data and instructions, SRAM designs cache.

  - Registers > L1 cache > L2 cache > memory > disk.

  - Memory in unit of byte or in unit of word.

- Cache: block is unit of copying.

  - Miss, miss penalty, and miss ratio - hit.

  - Temporal locality and spatial locality.

- DRAM: refresh to keep data. Store in bank, with several row, and each row has several column.

  - Pre to open / close a bank and act to access one or several (burst mode) rows (into buffer).

  - SDRAM (synchronous DRAM) adds a clock to synchronize with processor. Burst mode won't send multiple addresses.

    - DDR (double data rate) DRAM: transfer on both sides of clock.

- Flash: nonvolatile semiconductor storage.

  - Flash bits wears out, and use wear leveling to remap data to less used blocks.

- Disk: nonvolatile rotating magnetic storage.

  - Track, sector, queuing delay, seek (find track), rotational latency, data transfer, controller overhead.

Cache map

- Direct mapped cache: index, valid bit, tag, and data.

  - Index: from 0 to n - 1 of cache size, which is lower bits of memory address.

  - Tag: high order bits of memory address.

  - Valid bit: whether there is data in a location.

  - Data: data from memory.

- For unit of word memory, two bits of byte offset determines which byte to access.

- Larger block size: can be considered as more offset bits.

  - For 32 bits address, block size m word (m+2 bits), n bits of index:

- Cache miss: stall CPU pipeline and fetch block (instruction cache miss and data cache miss).

- Write through (update memory on cache change, use write buffer) and write back (update memory when leave cache, and log dirty bit).

- Write allocate: on write miss,

  - Write through (alternatives): allocate and fetch the block, or write around and don't fetch the block.

  - Write back: fetch the block, in fact much complex.

Review12. Memory Performance and Dependable Memory Hierarchy

2019年6月2日

10:07

Cache performance

- CPU time: program execution cycles and memory stall cycles.

  - I-cache (instruction cache) miss and D-cache (data cache) miss: all instructions can cause I-cache miss, while only lw / sw can cause D-cache miss.

Associative caches

- Group blocks into set: n-way associative n blocks in one set, fully associative n blocks in one set.

- N-way associative needs n comparator, and search n times, and n places to put blocks.

- Replacement policy: non-valid, then LRU (least-recently used), or random (approximately LRU for high associativity).

Multilevel caches

- Primary cache minimal hit time, L-2 cache low miss rate.

- Service accomplishment failure to service interruption, then restore.

  - Reliability: MTTF (mean time to failure).

  - Service interruption: MTTR (mean time to repair).

The Hamming SEC code

- Hamming distance: number of different bits between two bit pattern.

  - Distance 2 (parity code) SED, distance 3 SEC / DED.

- Encode and decode, error of parity bits can identify error position.

- SEC / DED code: add an additional parity bit for whole word pn.

  - Hamming distance becomes 4.

  - H is SEC parity bits: H zero pn even no error, pn odd pn error. H non-zero pn odd correct one error, pn even detect double error.

    - The additional bit is at the end of pattern. H is the calculated SEC parity bits after transfer (without consider additional bit).

    - Recalculate H and pn, according to even-odd decides error (in fact, consider H whether 0 in actual use).

  - Can detect at most 3 error, but don't know exactly which bit wrong, and would mix with SEC, then leads to wrong correction.

Review13. Virtual Memory

2019年6月2日

10:07

Virtual memory

- Programs share main memory and use private virtual memory.

  - VM page - block, VM page fault - miss.

- Page table: index PTE (page table entries) by virtual page number.

- Page table register in CPU and page table in memory.

- PTE: stores physical page number, referenced, dirty, valid…

- Page not present: PTE refer to location in swap space on disk.

- Replacement: LRU replacement using reference bit (use bit).

- Write: use write-back and set dirty bit in PTE.

Fast translation using TLB

- TLB (translation lookaside buffer): store recently used PTEs.

- Tag stores high bits of PTE virtual memory address.

- TLB miss: search PTE in memory.

  - Page in memory: load PTE and retry.

    - Hardware design more complicated page table, software raises a special exception.

  - Page not in memory: page fault, OS handles fetching page and restart the faulting instruction.

- TLB to find address and cache to find data.

Memory hierarchy

- Memory protection: tasks share parts of their virtual address spaces.

  - Privileged supervisor mode (kernel mode).

  - Privileged instructions and read only data (page table, state information).

  - Switch between kernel mode and user mode (system call exception).

- Block placement: direct map, n-way set associative, and fully associative.

- Finding a block: full lookup table of fully associative cost 0 comparison.

- Replacement on a miss: LRU or random.

  - Virtual memory uses LRU approximation with hardware support while cache uses either LRU or random.

- Write policy: write-through with write buffer, write-back with more states.

- Sources of misses: compulsory miss (cold start miss), capacity miss, conflict miss (collision miss).

Virtual machines

- Host computer emulates guest operating system and machine resources.

  - Improve isolation, avoid security and reliability problems, share resources.

  - Has some performance impact.

- Virtual machine monitor:

  - Maps virtual resources to physical resources.

  - Guest in user mode while VMM in privileged supervisor mode.

  - Guest OS different from host OS.

  - VMM handles real I/O devices and emulates for guest.

- Instruction set support.

Review14. Parallel Processors from Client to Cloud

2019年6月2日

10:07

Multiprocessors

- Parallel processing program: efficiently executed.

  - Hardware: serial and parallel.

  - Software: sequential and concurrent.

- Parallel and scaling: partition, coordination, and communication overhead.

  - Strong scaling: problem size fixed.

  - Weak scaling: problem size increases due to number of processors increases.

- Load balancing: each processor has same number of tasks.

Level parallelism

- Parallel processing: SISD (single instruction stream, single data stream), MIMD (multiple instruction multiple data), SPMD (single program multiple data), SIMD (single instruction multiple data), and vector.

  - SPMD: program on a multi-core processor, different processor execute on different sections of code.

  - SIMD: operate on vectors of data (data level parallelism), processors execute same instruction on different data address.

  - Vector processors: designed for vector operation, pipelined execution units.

- Multithreading: related to MIMD, multiple threads share a single processor.

  - Fine-grain multithreading: switch threads, interleave instruction execution, and execute other threads when one is stall.

  - SMT (simultaneous multithreading): multiple-issue dynamically scheduled processor.

Multicore microprocessors

- SMP (shared memory multiprocessor): efficiently programming on multiprocessor.

- Message parsing multiprocessors: each processor has private physical address space, and share messages via hardware.

- Loosely coupled clusters: network of independent computers.

- Cloud computer: WSC (warehouse scale computers), SaaS (software as a service), PMD (personal mobile device) or cloud running software.

- Data center: computers connected by off-the-shelf networking devices.

Other processor units

- GPU (graphic processing unit): highly data-parallel processing, oriented towards bandwidth.

- TPU (tensor processing unit): tensorflow platform.

- DPU (deep learning processing unit): FPGA-based processing unit.

- NPU (neural network processing unit): IBM TrueNorth.

- BPU (brain processing unit).

Review15. Final Review

2019年6月2日

10:07

The processor

- Units of CPU: PC, memory (instruction, data), registers, ALU.

  - MUX ternary operator, and ALU function.

  - R-type, lw / sw, J-type instructions.

- Control signals: RegDst, Branch, MemRead, ALUOp, MemWrite, ALUSrc, RegWrite.

The overview of pipeline

- Stages of pipeline: IF, ID, EXE, MEM, WB.

- Hazards: structure, data (forwarding, load-use), control.

Instruction-level parallelism

- Multiple issue: static (compiler), dynamic (hardware).

  - Speculation with reorder and buffer.

  - Exception handler.

- Multiple issue scheduling: replicate loop body.

Large and fast: exploiting memory hierarchy

- Memory: SRAM, DRAM, flash, disk.

- Cache map: index, valid bit, tag, data.

  - Write-through (buffer, allocation), write-back (complex, read textbook).

Memory performance and dependable memory hierarchy

- Instruction cache miss and data cache miss.

- Associative cache: direct map, n-way associative, fully associative.

  - Replacement: non-valid, LRU, random.

  - Parity bits and data bits, additional parity bit (SEC / DED).

Virtual memory

- VM page, VM page fault.

- Page table (memory): valid bit, dirty bit, reference bit, physical address.

- TLB (cache): valid bit, dirty bit, reference bit, tag, physical address.

- Memory hierarchy: direct map, n-way associative, fully associative (search n entries, full lookup table), .

  - Replacement: LRU approximate with hardware support.

  - Miss: compulsory miss, capacity miss, conflict miss.

Parallel processors from client to cloud

- Strong scaling and weak scaling, load balancing.

- Parallel processing (single core): SISD, MIMD, SPMD, SIMD, vector processor, multithreading (SMT).

  - Multicore: SMP, message parsing multiprocessor, cluster, cloud, data center.